# Mech 368 Lab 3

Thomas Richmond, 52228848

21 October 2021

# Exercise 1

Construction performed according to lab manual

# Exercise 2

## Goals

- Implement an op-amp

## Setup

The MCP6002 circuit diagram was obtained from the datasheet and is shown on the right.

To achieve V+=2,5V given an input voltage of 5V, we get the following relation:

VoutVin=R2R1+R2

2,5V5,0V=R2R1+R2

R1=R2

Let’s use a standard resistor value of R1=R2=100kΩ. The circuit is created as shown:

## Data

The output from Vout is measured using the oscilloscope. The trace is shown below.

The trace is very steady with a nominal voltage of 2,5V.

# Exercise 3

## Goals

- Understand the principle of a strain gauge using a mock-up

## Setup

## Data

The signals traces for V3 and V4 are very stable. At a point in time, we see the following output:

We see the signal is within one major division, corresponding to a difference in signals of

1mV≤V3-V4=3mV≤10mV

which is compliant with the requirements of this exercise.

# Exercise 4

## Goals

- Analyze a Wheatstone bridge

## Setup

The questions posed in the exercise are answered below:

Work shown on next page

## Data

The signal amplification is calculated from the following trace, given the blue trace is V1-V2 for the strain gauge and the yellow trace is the amplifier output voltage Vout.

G=Vout-VrefVin+-Vin-

Using the voltmeter function of the AD2, we find the following data:

| Data | Value [mV] |
| --- | --- |
| Vout | 3974 |
| Vref | 2502 |
| Vin+ | 2488 |
| Vin- | 2495 |

Plugging these values into the equation above yields G=210,29 which is very close to our target value!

The no-load and maximum-load outputs are measured to be 2604mV and 3031mV, respectively. The difference in measurements is 427mV, which is commensurate with the values in the lab report.

# Exercise 5

## Goals

- Implement an offset amplifier to generate a reference voltage

## Setup

The circuit was tuned from the initial setup above such that the final nominal resistance of resistor series R13 is  108,7kΩ.

The final circuit setup is as shown:

With annotations:

## Data

The no-load and maximum-load outputs are measured to be 2498mV and 460mV, respectively. This commensurate with the respective datapoints of 2500mV and 500mV as listed in the lab report.

# Exercise 6

## Goal

- Develop a program to visualize

## Setup

The code used was largely the same as from Lab 2 Exercise 6, with some minor modifications.

## Data

The output for the program is seen below. The graph contains some troughs, corresponding to probing of the load cell with my finger.

The code for Exercise 6 is further developed in Exercise 8. Refer to Appendix A for the code for Exercise 8 (code contained for this exercise is embedded therein)

# Exercise 7

## Goal

- Fit a trendline to a load cell to convert voltage to load

## Setup

A trendline is developed by placing a variety of weights onto the load cell and recording the voltage output from the offset amplifier. This is performed using Excel.

## Data

The datapoints are plotted on a scatterplot and a trendline is fitted. A slope of -0,9906gmV is obtained.

| Load [g] | Voltage [mV] |
| --- | --- |
| 0 | 2473 |
| 50 | 2418 |
| 100 | 2369 |
| 200 | 2270 |
| 300 | 2167 |
| 500 | 1965 |
| 1000 | 1461 |
| 1500 | 956 |
| 2000 | 452 |

# Exercise 8

## Goal

- Develop a UI to display load from a load cell

- Implement a stability detection algorithm

## Setup

The code from Exercise 6 is modified slightly to incorporate the following features:

- The average voltage is multiplied by the slope above to generate a load reading

- Tare functionality is added in two ways:

  - A tare button is added to the UI, allowing for the user to tare the scale at any time

  - The tare is automatically set the first time a signal is read by the program

- A stability detection algorithm is implemented; the algorithm generates a standard deviation over the last five hundred milliseconds, and if it is below 10 (empirically determined) then the reading is considered stable

## Data

The program functions very well. When the load cell is loaded to 500g the load is reported to be between 498 and 502 grams, falling within 1% error.

# Appendix A: Exercise 8 Code

using System; using System.IO; using System.IO.Ports; using System.Windows.Forms; using System.Collections.Generic; using System.Collections.Concurrent; using System.Windows.Forms.DataVisualization.Charting; namespace Mech368Lab2E6 { public partial class Mech368L3E6 : Form { enum AD2Token { PACKET_HEADER = 0xFF, STREAM_REQUEST_ACCELEROMETER = 'A', STREAM_REQUEST_THERMISTOR = 'c', LED5_ON = '5', LED5_OFF = '%' } private ConcurrentQueue<int> streamBuffer; private SerialPort port; private StreamWriter streamWriter; private Series loadvoltages; private Series loads; private double load = 0; private double tare = double.NaN; public Mech368L3E6() { InitializeComponent(); InitializeListeners(); this.streamBuffer = new ConcurrentQueue<int>(); this.port = this.BindSerial(); this.port.Write(((char)AD2Token.STREAM_REQUEST_THERMISTOR).ToString()); this.loadvoltages = new Series("Load Voltages"); this.loadvoltages.ChartType = SeriesChartType.FastLine; this.loadvoltages.YValueType = ChartValueType.Double; //plot.Series.Add(this.loadvoltages); this.loads = new Series("Load"); this.loads.ChartType = SeriesChartType.FastLine; this.loads.YValueType = ChartValueType.Double; plot.Series.Add(this.loads); //this.tareBtn.Visible = false; //this.stabilityLabel.Visible = false; //this.loadTB.Visible = false; //this.label3.Visible = false; //this.label4.Visible = false; } private void InitializeListeners() { this.filepathTB.Click += this.ConfigureOutputStream; this.savetofileCB.CheckedChanged += this.ToggleFilewriter; this.processLoop.Tick += this.ProcessDatastream; this.processLoop.Tick += this.DetectStability; this.tareBtn.Click += this.SetTare; } private SerialPort BindSerial() { SerialPort p = new SerialPort() { PortName = "COM3", BaudRate = 9600, Parity = Parity.None, DataBits = 8, StopBits = StopBits.One, Handshake = Handshake.None }; p.DataReceived += this.CaptureDatastream; p.Open(); return p; } private void ConfigureOutputStream(object sender, EventArgs e) { SaveFileDialog sfd = new SaveFileDialog(); if (sfd.ShowDialog() == DialogResult.OK) filepathTB.Text = sfd.FileName; } private void ToggleFilewriter(object sender, EventArgs e) { if (this.streamWriter == null) if (this.filepathTB.Text.Length > 0) this.streamWriter = new StreamWriter(this.filepathTB.Text); else MessageBox.Show("Invalid filename!"); else { this.streamWriter.Close(); this.streamWriter = null; } } private void OnFormClosing(object sender, EventArgs e) { if (this.port != null && this.port.IsOpen) this.port.Close(); if (this.streamWriter != null) this.streamWriter.Close(); } private void CaptureDatastream(object sender, EventArgs e) { int next; // Byte buffer try { while ((next = this.port.ReadByte()) != -1) this.streamBuffer.Enqueue(next); // ConcurrentQueue buffer implementation } catch { /*Ignored!*/ } } Queue<int> circularBuffer = new Queue<int>(); private void ProcessDatastream(object sender, EventArgs e) { void EnqueueCircular(Queue<int> queue, int next, int max) { queue.Enqueue(next); if (queue.Count > max) queue.Dequeue(); } int AverageQueue(Queue<int> queue, int offset = 0) { float avg = 0; foreach (int val in queue) avg += val - offset; avg /= (float)queue.Count; return (int) Math.Round(avg); } int signal = -1; while (this.streamBuffer.TryDequeue(out int next)) { if (next == (int)AD2Token.PACKET_HEADER) { if (signal == -1) { signal = 0; continue; } double discretized = signal / ((double)(2 << 9)); int nextVoltage = (int) (discretized * 3600); EnqueueCircular(this.circularBuffer, nextVoltage, 50); int avgVoltage = AverageQueue(this.circularBuffer); voltageTB.Text = nextVoltage.ToString(); voltageAvgTB.Text = avgVoltage.ToString();// avgVoltage.ToString(); this.loadvoltages.Points.AddY(nextVoltage); double nextLoad = avgVoltage * -0.9906; if (double.IsNaN(this.tare)) this.tare = nextLoad; nextLoad -= tare; loadTB.Text = nextLoad.ToString(); this.loads.Points.AddY(load); if (this.streamWriter != null) this.streamWriter.WriteLine( $"{DateTime.Now.ToString("hh.mm.ss.ffffff")}, " + $"{nextVoltage}, " + $"{avgVoltage}, " + nextLoad.ToString("0.##")); this.load = nextLoad; signal = 0; continue; } else if (signal == -1) // Haven't captured a packet header continue; else // Assemble bytes signal = (signal << 5) + next; } } private void DetectStability(object sender, EventArgs e) { double SumSquaresQueue(Queue<int> queue) { float summed = 0; foreach (int val in queue) summed += val * val; return summed; } double AverageQueue(Queue<int> queue, int offset = 0) { float avg = 0; foreach (int val in queue) avg += val - offset; avg /= (float)queue.Count; return avg; } double sum = SumSquaresQueue(this.circularBuffer); double mean = AverageQueue(this.circularBuffer); double stdev = Math.Sqrt(sum / ((double)this.circularBuffer.Count) - mean * mean); //this.filepathTB.Text = stdev.ToString(); this.stabilityLabel.Text = stdev < 10 ? "STABLE" : "UNSTABLE"; } private void SetTare(object sender, EventArgs e) { this.tare = this.load + this.tare; } } }